CommClient

LAN/WiFi Communication Platform

System Test Plan

Document Version: 1.0

Date: April 2026

Classification: Internal

Prepared for production deployment validation on isolated LAN/WiFi networks

# Table of Contents

# 1. Introduction

This document defines the complete system-level test plan for the CommClient LAN/WiFi-only communication platform. The plan covers end-to-end functional validation, failure resilience, and performance benchmarks across all communication modes: private chat, group messaging, audio/video calls, screen sharing, presence, and file transfer.

All tests are designed to execute on an isolated local network (WiFi or Ethernet LAN) with no internet dependency. The platform architecture is: Electron 30 + React 18 + Zustand (desktop client), FastAPI + python-socketio + SQLAlchemy async + aiosqlite (server), WebRTC mesh topology for media (max 8 participants per call).

## 1.1 Scope

In scope: functional correctness of all 12 core scenarios, fault tolerance under network disruption, server crash recovery, data persistence, presence accuracy, concurrent room isolation, and baseline performance metrics.

Out of scope: load testing beyond 20 concurrent users, penetration testing (covered in SECURITY-ARCHITECTURE.md), UI/UX usability testing, accessibility compliance, cross-platform testing (macOS/Linux).

## 1.2 Test Environment

| Component | Specification |
| --- | --- |
| Network | Isolated WiFi router (802.11ac or better) OR Ethernet switch, no internet uplink |
| Server Machine | Windows 10/11, 8 GB RAM min, Python 3.11+, port 3000 available |
| Client Machines | 2-20 Windows 10/11 devices on the same subnet (192.168.x.0/24) |
| Server Build | CommClient-Server.exe (PyInstaller single-folder bundle) |
| Client Build | CommClient Setup.exe (NSIS installer, Electron + Vite) |
| Audio/Video HW | Each client device must have microphone + camera (or virtual devices for automation) |
| Discovery | mDNS/Zeroconf on UDP port 41234 |
| Database | SQLite (aiosqlite), auto-created at %APPDATA%/CommClient/data/commclient.db |
| Browser Fallback | None (Electron-only desktop app) |

## 1.3 Conventions

- Device A, Device B, Device C... Device T = individual test machines (up to 20)

- Server = the machine running CommClient-Server.exe on port 3000

- User-A, User-B... = registered accounts on the respective devices

- RTT = Round-trip time measured via Socket.IO ping

- SDP = Session Description Protocol (WebRTC offer/answer)

- ICE = Interactive Connectivity Establishment (WebRTC candidate exchange)

# 2. Full Test Matrix

The matrix below enumerates every test scenario, its category, device count, priority, and the interfaces exercised. Each row maps to a detailed test procedure in Section 3.

| ID | Scenario | Category | Devices | Priority | Interfaces |
| --- | --- | --- | --- | --- | --- |
| TC-01 | Private Text Chat (2 devices) | Messaging | 2 | P0 | REST /auth, Socket message:send, message:delivered, message:read |
| TC-02 | Private Audio Call (2 devices) | Calls | 2 | P0 | Socket call:initiate, call:accept, WebRTC offer/answer/ICE |
| TC-03 | Private Video Call (2 devices) | Calls | 2 | P0 | Socket call:initiate, call:accept, WebRTC offer/answer/ICE (video tracks) |
| TC-04 | Private Screen Sharing (2 devices) | Sharing | 2 | P1 | Socket presenter:start, presenter:stop, WebRTC getDisplayMedia |
| TC-05 | Group Messaging (3-20 devices) | Messaging | 3-20 | P0 | REST /channels, Socket message:send, channel:join, channel:leave |
| TC-06 | Group Audio/Video Call (3-8 devices) | Calls | 3-8 | P0 | Socket call:initiate (group), call:join_group, WebRTC mesh N*(N-1)/2 peers |
| TC-07 | Group Screen Sharing | Sharing | 3-8 | P1 | Socket presenter:start, presenter:stop, WebRTC replaceTrack |
| TC-08 | WiFi Interruption Recovery | Resilience | 2+ | P0 | Socket reconnect, MessageQueue offline buffer, presence:user_offline/online |
| TC-09 | Server Restart Recovery | Resilience | 2+ | P0 | Server health /docs, Socket reconnect, REST /auth/refresh, DB persistence |
| TC-10 | Message Persistence | Data | 2 | P1 | REST /messages, SQLite query, Socket message:history |
| TC-11 | Presence Updates | Presence | 3+ | P0 | Socket presence:user_online, presence:user_offline, presence:online_list |
| TC-12 | Multiple Simultaneous Rooms | Isolation | 4+ | P1 | Socket channel:join (multiple), message:send isolation, call isolation |

# 3. Manual Test Procedures

Each procedure below lists preconditions, step-by-step actions, and the expected result at each step. Testers should mark each step PASS or FAIL and note any deviation.

## 3.1 TC-01: Private Text Chat (2 Devices)

### Preconditions

- Server running on Device S, accessible at http://<server-ip>:3000

- User-A registered and logged in on Device A

- User-B registered and logged in on Device B

- Both clients show green socket connection indicator

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A opens the user list and selects User-B to start a private conversation | A DM channel is created (or existing one opened). REST POST /channels returns 200 or existing channel ID. |
| 2 | User-A types 'Hello from A' and presses Send | Message appears in User-A's chat with status 'sent'. Socket event message:send emitted. |
| 3 | Observe Device B within 2 seconds | Message 'Hello from A' appears in User-B's chat. Socket event message:new received. Delivery indicator shows 'delivered'. |
| 4 | User-B reads the message (chat window is in focus) | User-A sees 'read' status on the message. Socket event message:read received. |
| 5 | User-B replies 'Hello from B' | Message appears on Device B as 'sent', then 'delivered' on Device A within 2 seconds. |
| 6 | User-A sends a 5 MB image file via the attachment button | File uploads via REST POST /files/upload (200). Thumbnail appears in chat. User-B can download the full file via GET /files/download/<id>. |
| 7 | User-A sends 50 rapid messages (paste + enter) | All 50 messages arrive on Device B in correct order. No duplicates. Rate limiter does NOT block (under 15 msg/sec threshold). |
| 8 | User-A sends message #51 within the same second window if rate limit was approached | If rate limit triggers, User-A receives a rate_limited error event. Message is NOT delivered to User-B. |
| 9 | Both users close and reopen the app | Chat history is intact. All 50+ messages visible. Read receipts preserved. |

## 3.2 TC-02: Private Audio Call (2 Devices)

### Preconditions

- Same as TC-01, plus: both devices have working microphones and speakers/headphones

- No other active calls in the system

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A clicks the audio call button on User-B's DM channel | Socket emits call:initiate with type='audio'. User-A sees 'Calling...' UI. Server creates call record with status='ringing'. |
| 2 | Observe Device B within 3 seconds | Device B shows incoming call notification with User-A's display name. Ringtone plays. Socket event call:incoming received. |
| 3 | User-B clicks 'Accept' | Socket emits call:accept. Server updates call status to 'active'. Both clients enter call UI. |
| 4 | Observe WebRTC negotiation (browser devtools or logs) | SDP offer sent from initiator, answer from acceptor. ICE candidates exchanged. DTLS handshake completes. RTCPeerConnection state = 'connected'. |
| 5 | User-A speaks into microphone | User-B hears User-A's voice with less than 300ms perceived latency on LAN. Audio meters show activity on both sides. |
| 6 | User-B speaks into microphone | User-A hears User-B's voice. Full-duplex confirmed (both can speak simultaneously without clipping). |
| 7 | User-A mutes microphone via UI toggle | User-A's audio track is disabled. User-B's audio meter for User-A drops to zero. Mute indicator appears on User-B's UI. |
| 8 | User-A unmutes | Audio resumes. User-B hears User-A again within 1 second. |
| 9 | User-A clicks 'End Call' | Socket emits call:end. Both clients return to chat view. Server updates call status to 'ended'. Call duration logged. |
| 10 | Verify call log persistence | REST GET /calls/history shows the completed call with correct duration, participants, and timestamps. |

## 3.3 TC-03: Private Video Call (2 Devices)

### Preconditions

- Same as TC-02, plus: both devices have working cameras

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A clicks the video call button on User-B's DM channel | Socket emits call:initiate with type='video'. Camera permission requested and granted. Local video preview appears. |
| 2 | User-B accepts the call | Both clients enter video call UI. SDP negotiation includes video tracks (VP8/VP9/H.264). ICE negotiation completes. |
| 3 | Verify video rendering on both devices | User-A sees User-B's camera feed. User-B sees User-A's camera feed. Video resolution is at least 640x480 on LAN. |
| 4 | User-A disables camera via UI toggle | User-A's video track is stopped. User-B sees a placeholder/avatar instead of video. Audio remains active. |
| 5 | User-A re-enables camera | Video resumes within 2 seconds. User-B sees User-A's camera feed again. |
| 6 | User-B switches to audio-only mid-call | User-B's video track stops. User-A sees placeholder for User-B. Audio continues bidirectionally. |
| 7 | Call runs for 5 minutes continuously | No frame drops, no audio desync, no ICE restart triggered. RTCPeerConnection remains 'connected'. Memory usage stable (no leaks). |
| 8 | User-B ends the call | Both clients return to chat. Call log shows type='video' with accurate duration. |

## 3.4 TC-04: Private Screen Sharing (2 Devices)

### Preconditions

- Active audio or video call between User-A and User-B (TC-02 or TC-03 completed)

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A clicks 'Share Screen' during an active call | Browser's getDisplayMedia() picker appears. User-A selects a screen/window. Socket emits presenter:start. |
| 2 | Observe Device B within 3 seconds | User-B sees User-A's shared screen in the call UI. Screen share track replaces or supplements the video track. |
| 3 | User-A moves windows, types text on shared screen | Updates are visible on Device B in real-time with less than 500ms visual latency on LAN. |
| 4 | User-A stops screen sharing via UI button | Socket emits presenter:stop. User-B's view reverts to camera feed (if video call) or audio-only UI. |
| 5 | User-B initiates screen share (role swap) | User-A now sees User-B's screen. Confirms bidirectional screen sharing works. |
| 6 | User-B stops sharing and ends the call | Clean teardown: presenter state cleared, call ended, call log persisted. |

## 3.5 TC-05: Group Messaging (3-20 Devices)

### Preconditions

- Server running. 3 to 20 users registered and logged in on separate devices.

- User-A creates a group channel named 'Test Group' and invites all other users.

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A creates group channel via REST POST /channels with type='group' | Channel created with User-A as owner. Channel ID returned. All invited users receive channel:invite socket event. |
| 2 | All invited users accept and join the channel | Each user's client emits channel:join. Server confirms membership. All users see the channel in their sidebar. |
| 3 | User-A sends 'Welcome everyone' to the group | Message appears on ALL member devices within 3 seconds. message:new event received by each member. |
| 4 | User-C (any non-creator) sends a reply | Reply visible to all members including User-A. Correct ordering maintained. |
| 5 | All N users send a message simultaneously (coordinated) | All N messages arrive on all devices. No message loss. Order may vary but all N messages are present on every client within 5 seconds. |
| 6 | User-B leaves the channel | User-B removed from member list. Other members see 'User-B left' system message. User-B no longer receives group messages. |
| 7 | User-A sends a message after User-B left | Message delivered to all remaining members. User-B does NOT receive it. |
| 8 | User-A sends a 10 MB file to the group | File uploaded successfully. All members can download. Thumbnail visible in chat for image files. |
| 9 | Verify message count at 20 users with 100 messages each | Total 2000 messages stored and retrievable. Pagination works via REST GET /messages?channel_id=X&limit=50&offset=0. |

## 3.6 TC-06: Group Audio/Video Call (3-8 Devices)

### Preconditions

- Group channel exists with 3 to 8 members (TC-05 precondition).

- All devices have microphone + camera.

- MAX_CALL_PARTICIPANTS = 8 (server config).

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A initiates a group call in the channel | Socket emits call:initiate with type='group'. Server creates call with status='ringing'. All channel members receive call:incoming. |
| 2 | Users B, C, D join the call sequentially | Each join triggers call:join_group. Server adds participant atomically (asyncio.Lock). Each new joiner establishes WebRTC peer connections with ALL existing participants (mesh: N*(N-1)/2 connections). |
| 3 | With 4 participants, verify mesh topology | 6 peer connections total (4*3/2). Each participant has 3 RTCPeerConnection objects. All audio/video streams active. |
| 4 | User-E (5th participant) joins | 10 peer connections total. User-E establishes 4 new connections. Existing participants each add 1 new connection. All media flows correctly. |
| 5 | All participants speak in round-robin | Each participant's voice is audible to all others. No echo, no feedback loops. Audio mixing handled by browser. |
| 6 | User-C enables video mid-call | User-C's video track added via renegotiation. All other participants see User-C's video. SDP renegotiation uses perfect negotiation pattern (no glare). |
| 7 | User-B disconnects unexpectedly (close app) | Server detects disconnect. Socket disconnect handler cleans up: call_service.leave_call, presenter_service.remove_participant. Other participants receive call_participant_left with reason='disconnect'. Their peer connections to User-B are closed. |
| 8 | Attempt to add a 9th participant when MAX_CALL_PARTICIPANTS=8 | Server rejects with error 'Call is full'. 9th user receives call:error event. Existing call unaffected. |
| 9 | User-A (initiator) ends the group call | All participants receive call:ended. All peer connections closed. Call log persisted with participant list and durations. Presenter state cleaned up. |

## 3.7 TC-07: Group Screen Sharing

### Preconditions

- Active group call with 3+ participants (TC-06 steps 1-3 completed).

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A clicks 'Share Screen' during the group call | getDisplayMedia picker appears. User-A selects screen. Socket emits presenter:start with call_id and user_id. |
| 2 | Observe all other participants within 3 seconds | All participants see User-A's screen share. presenter:started event received by all. UI shows User-A as presenter. |
| 3 | User-B attempts to share screen while User-A is sharing | Behavior depends on implementation: either User-A's share stops and User-B takes over, or User-B is denied with an error. Document observed behavior. |
| 4 | User-A stops sharing | presenter:stop emitted. All participants revert to normal call view. Presenter state cleared in presenter_service. |
| 5 | User-C starts sharing immediately after | Seamless handover. All participants see User-C's screen within 3 seconds. |
| 6 | New participant (User-D) joins the call while User-C is sharing | User-D receives presenter state on join. User-D sees User-C's screen share without requiring User-C to restart sharing. |

## 3.8 TC-08: WiFi Interruption Recovery

### Preconditions

- Active private chat between User-A and User-B with message history.

- Both connected via WiFi (not Ethernet) for easy disruption testing.

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A disables WiFi on Device A (toggle off adapter) | Device A loses network. Socket.IO connection drops. Client shows 'disconnected' or 'reconnecting' indicator. |
| 2 | User-B sends 'Are you there?' while User-A is offline | Message stored on server (SQLite). User-B sees 'sent' status. Server processes message:send but cannot deliver to User-A. |
| 3 | Wait 30 seconds, then re-enable WiFi on Device A | Device A reconnects to WiFi. Socket.IO auto-reconnect triggers. JWT re-authentication via auth payload on reconnect. |
| 4 | Observe Device A after reconnection (within 10 seconds) | Socket reconnect succeeds. presence:user_online broadcast. User-A receives missed message 'Are you there?' with correct timestamp. |
| 5 | User-A sends a message while offline (WiFi disabled) | Message is queued in MessageQueue (offline buffer). UI shows 'pending' or 'queued' status. |
| 6 | Re-enable WiFi on Device A | Queued message is sent automatically on reconnect. User-B receives it. Message ordering is preserved. |
| 7 | Disconnect User-A during an active audio call | Call is disrupted. Server disconnect handler fires: call_service.leave_call called. User-B receives call_participant_left with reason='disconnect'. After reconnect, User-A must initiate a NEW call (auto-rejoin not supported). |
| 8 | Rapid on/off WiFi toggling (5 times in 10 seconds) | Socket.IO reconnect backoff handles gracefully. No duplicate connections. Connection count does not exceed MAX_CONNECTIONS_PER_USER (5). Presence updates are correct (online after final reconnect). |

## 3.9 TC-09: Server Restart Recovery

### Preconditions

- 2+ clients connected and authenticated.

- Active chat with message history in SQLite database.

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | Kill the server process (taskkill /pid /t /f or close the terminal) | All clients lose socket connections simultaneously. Clients show 'disconnected' indicator. |
| 2 | Observe client behavior for 30 seconds (server still down) | Clients attempt Socket.IO reconnection at increasing intervals (1s, 2s, 4s...). No crash. UI remains functional for viewing cached messages. |
| 3 | Restart the server (run CommClient-Server.exe) | Server starts, binds to port 3000, health check endpoint /docs responds. |
| 4 | Observe clients within 30 seconds of server restart | Socket.IO reconnect succeeds. JWT is re-validated. If access token expired, automatic token refresh via REST POST /auth/refresh. |
| 5 | Verify message history after reconnection | All messages sent before server crash are intact in SQLite. Clients load history via REST GET /messages. No data loss. |
| 6 | Verify presence state after reconnection | All reconnected users show as 'online'. presence:online_list is accurate. Users who did not reconnect show as 'offline'. |
| 7 | Send new messages after server restart | Full messaging functionality restored. New messages delivered correctly. Read receipts work. |
| 8 | Verify active calls were properly terminated | Any calls that were active during crash are now 'ended' (orphaned call cleanup). Users must initiate new calls. |

## 3.10 TC-10: Message Persistence

### Preconditions

- Server running with SQLite database.

- At least 2 users with an established DM channel.

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | Send 100 text messages in a DM channel from User-A to User-B | All 100 messages stored in SQLite messages table. REST GET /messages?channel_id=X&limit=100 returns all 100. |
| 2 | Close the client app on both devices completely | Apps exit cleanly. No background processes. |
| 3 | Reopen both client apps and log in | Session restored (safeStorage tokens). Socket reconnects. Chat history loads from server. |
| 4 | Verify all 100 messages are present with correct content, order, and timestamps | Message list matches original send order. Timestamps are accurate (within 1 second of original send). Content is byte-for-byte identical. |
| 5 | Verify read receipts persisted | Messages User-B read before closing still show 'read' status on User-A's device. |
| 6 | Send a message with Unicode content (Arabic, emoji, CJK) | Unicode preserved correctly in storage and retrieval. No mojibake. Example: send 'مرحبا 🌍 こんにちは' and verify exact match. |
| 7 | Stop the server, copy the SQLite file, restart server | Database file is a valid SQLite3 database. Can be opened with any SQLite tool. Schema is intact. Data is queryable. |
| 8 | Delete a message from User-A (if supported) | Message marked as deleted in DB. Other users see deletion indicator. Message content is no longer retrievable. |

## 3.11 TC-11: Presence Updates

### Preconditions

- Server running. 3+ users registered.

- All users initially logged out.

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-A logs in on Device A | Server emits presence:user_online with user_id=A. No other users to receive it yet. |
| 2 | User-B logs in on Device B | User-B receives presence:online_list containing [User-A]. User-A receives presence:user_online for User-B. Both show as online. |
| 3 | User-C logs in on Device C | User-C receives presence:online_list containing [User-A, User-B]. Users A and B each receive presence:user_online for User-C. |
| 4 | User-A logs out (click logout) | Server revokes session, emits presence:user_offline for User-A. Users B and C see User-A go offline. User-A's status in DB = 'offline', last_seen updated. |
| 5 | User-B closes app without logout (force close) | Server detects socket disconnect. After all of User-B's sockets close, presence_service.disconnect returns User-B's ID. Server emits presence:user_offline for User-B. User-C sees User-B go offline. |
| 6 | User-A logs in from TWO devices simultaneously | Both connections authenticated. Connection count = 2. User-A shows as online (single presence, multiple sockets). Other users see one online event. |
| 7 | User-A disconnects ONE of the two devices | Connection count decreases to 1. User-A remains online (still has one active socket). No offline event broadcast. |
| 8 | User-A disconnects the LAST device | Connection count = 0. presence:user_offline broadcast. User-A now offline. |
| 9 | Rapidly connect/disconnect User-A 10 times in 5 seconds | Presence updates are atomic (asyncio.Lock in presence_service). Final state is correct (online if connected, offline if not). No ghost presences. |

## 3.12 TC-12: Multiple Simultaneous Rooms

### Preconditions

- 4+ users logged in. Two separate group channels created: 'Room-Alpha' (Users A, B, C) and 'Room-Beta' (Users A, C, D).

- User-A and User-C are members of BOTH rooms.

### Steps

| Step | Action | Expected Result |
| --- | --- | --- |
| 1 | User-B sends 'Hello Alpha' in Room-Alpha | Users A and C receive the message in Room-Alpha. User-D does NOT receive it (not a member). |
| 2 | User-D sends 'Hello Beta' in Room-Beta simultaneously | Users A and C receive the message in Room-Beta. User-B does NOT receive it (not a member). |
| 3 | User-A verifies both messages arrived in the correct rooms | Room-Alpha shows 'Hello Alpha'. Room-Beta shows 'Hello Beta'. No cross-room contamination. |
| 4 | User-A initiates a call in Room-Alpha | Only Room-Alpha members (A, B, C) receive call:incoming. User-D (Room-Beta only) does NOT receive call notification. |
| 5 | While call is active in Room-Alpha, User-D sends a message in Room-Beta | Message delivered to Users A and C in Room-Beta. User-A can receive messages while in a call. Call audio is unaffected. |
| 6 | User-C shares screen in Room-Alpha call while also receiving Room-Beta messages | Screen share works in Room-Alpha. Room-Beta messages continue arriving. No interference between the two rooms. |
| 7 | Verify channel isolation: User-D attempts to send message to Room-Alpha (via crafted socket event) | Server validates channel membership. Message rejected because User-D is not a member. Error returned to User-D. |

# 4. Automated Test Ideas

The following describes automation strategies for each test category, including recommended tools, framework patterns, and specific tests suitable for CI integration.

## 4.1 REST API Automation (pytest + httpx)

All REST endpoints can be tested headlessly using pytest-asyncio with httpx as the async HTTP client. These tests run against a real FastAPI instance with an in-memory SQLite database.

| Test Suite | Tests | Automation Approach |
| --- | --- | --- |
| Auth Registration | Valid registration, duplicate username, weak password, invalid username chars, SQL injection in username | httpx POST /auth/register with parametrized fixtures |
| Auth Login | Valid login, wrong password, non-existent user, rate limiting (11 requests), account lockout (16 requests) | pytest-asyncio with rate limit state reset between tests |
| Token Refresh | Valid refresh, expired refresh, revoked refresh, wrong token type | JWT fixture factory with configurable expiry |
| Channel CRUD | Create DM, create group, invite member, remove member, channel listing, duplicate channel prevention | Async session fixtures with rollback |
| Message CRUD | Send message, list messages with pagination, message ordering, Unicode content, max length validation | Bulk insert fixtures + parametrized pagination tests |
| File Upload/Download | Upload valid file, oversized file (>100MB), unauthorized download, thumbnail generation, Content-Disposition header check | multipart/form-data fixtures with tempfile.NamedTemporaryFile |
| Authorization Matrix | Non-member file access, non-member message send, non-member channel read | Matrix of (user_role, endpoint, expected_status) tuples |

## 4.2 Socket.IO Automation (python-socketio client)

Socket events can be tested using python-socketio's AsyncClient connecting to the running server. Each test authenticates with a valid JWT and validates event payloads.

| Test Suite | Tests | Automation Approach |
| --- | --- | --- |
| Presence Events | Connect broadcast, disconnect broadcast, online list accuracy, multi-device presence | Two AsyncClient instances per test, asyncio.Event for synchronization |
| Messaging Events | message:send delivery, message:delivered ack, message:read receipt, offline message queue | Publish from client A, assert receipt on client B with 5-second timeout |
| Call Signaling | call:initiate, call:accept, call:reject, call:end, unauthorized signal relay | State machine assertions: ringing -> active -> ended |
| Rate Limiting | Per-event limit (>15/sec), global aggregate limit (>100/sec), connection limit (>5 concurrent) | Burst loop with counter, assert rate_limited event received |
| Reconnection | Token re-auth on reconnect, presence restoration, missed event delivery | Disconnect/reconnect with sleep, verify state consistency |

## 4.3 WebRTC E2E Automation (Playwright + Electron)

Full end-to-end WebRTC testing requires real browser contexts. Use Playwright with Electron support to launch multiple client instances.

| Test Suite | Tests | Automation Approach |
| --- | --- | --- |
| Audio Call E2E | Call initiation, acceptance, media flow, hangup, call log creation | Playwright Electron: launch 2 app instances, getPermission('microphone'), check RTCPeerConnection.connectionState |
| Video Call E2E | Video track negotiation, camera toggle, resolution verification | Playwright: capture video element dimensions, verify non-black frames via canvas snapshot |
| Screen Share E2E | getDisplayMedia flow, presenter state, stop sharing | Playwright: mock getDisplayMedia with a canvas stream, verify receiver gets frames |
| Group Call E2E | Sequential join, mesh verification, participant leave | N Playwright instances, verify peer count = N-1 on each |
| Network Resilience E2E | WiFi drop simulation, reconnect, message queue flush | Playwright: page.context.setOffline(true/false) to simulate disconnection |

## 4.4 Database Validation (SQLAlchemy + aiosqlite)

Direct database assertions verify data integrity beyond what the API exposes.

- Message ordering: SELECT messages ORDER BY created_at WHERE channel_id=X, verify no gaps in sequence.

- Session hygiene: After logout, verify UserSession.is_active = False and no orphaned active sessions.

- Call log completeness: After call:end, verify call_logs has correct started_at, ended_at, participant list.

- File metadata integrity: After upload, verify file record matches actual file on disk (size, mime_type, filename).

- Presence consistency: After all clients disconnect, verify zero entries in presence_service's in-memory store.

## 4.5 CI Integration

Recommended CI pipeline stages:

- Unit tests: pytest with mocked dependencies (fastest, runs on every commit)

- API integration tests: pytest + httpx against real FastAPI + in-memory SQLite (runs on every PR)

- Socket integration tests: python-socketio AsyncClient against real server (runs on every PR)

- E2E tests: Playwright Electron against packaged build (runs nightly or pre-release)

- Regression: full test suite run before every release tag

# 5. Failure Scenarios

This section catalogs failure modes, their expected system behavior, and the recovery mechanism. Each scenario should be explicitly tested.

| ID | Failure Scenario | Expected Behavior | Recovery | Severity |
| --- | --- | --- | --- | --- |
| F-01 | Server process killed during active call | All call participants lose connection. disconnect handler fires asynchronously (may not complete). Call state left in 'active' in memory (lost). | Clients reconnect after server restart. Orphaned calls are not in new server memory. Users must initiate new calls. | HIGH |
| F-02 | Client WiFi adapter disabled mid-message-send | Socket.IO disconnects. Unsent message is queued in MessageQueue offline buffer. UI shows 'pending' status. | On WiFi restore, Socket.IO reconnects. MessageQueue flushes pending messages. Messages arrive in correct order. | MEDIUM |
| F-03 | SQLite database file locked (concurrent access from external tool) | aiosqlite raises OperationalError: database is locked. API returns 500. | Server retries with WAL mode. External tool must release lock. Restart server if persistent. | HIGH |
| F-04 | Client sends message to non-existent channel | Server validates channel_id. Returns error: channel not found. Message is NOT stored. | Client should refresh channel list. If channel was deleted, remove from local UI. | LOW |
| F-05 | JWT access token expires during active session | Next REST API call returns 401. Socket remains connected (socket auth uses separate token validation). | Client intercepts 401, calls POST /auth/refresh with refresh_token. New access_token obtained. Original request retried. | MEDIUM |
| F-06 | Refresh token stolen and used from another device | Attacker can issue new access tokens. Original user's refresh token still valid until rotated. | On next legitimate refresh, old token is revoked (rotation). Both sessions invalidated if suspicious. Audit log captures IP mismatch. | CRITICAL |
| F-07 | All 8 participants in a call simultaneously toggle video on | 8 SDP renegotiations triggered simultaneously. WebRTC perfect negotiation pattern handles glare. | Renegotiations resolve via polite/impolite role assignment. May take 2-5 seconds for all video to stabilize. | MEDIUM |
| F-08 | Server disk full (SQLite writes fail) | INSERT operations fail. API returns 500. Messages are lost if not queued client-side. | Free disk space. Restart server. Client-side MessageQueue provides buffer for messages sent during outage. | HIGH |
| F-09 | User opens 6 simultaneous socket connections (above limit) | 6th connection is rejected by socket_rate_limiter.can_connect(). Connection refused with 'Too many concurrent connections'. | User must close existing connections. Rate limiter tracks count via record_connect/record_disconnect. | LOW |
| F-10 | Network partition: half the clients on subnet A, half on subnet B | Clients on same subnet as server remain connected. Clients on other subnet lose connection. | When partition heals, disconnected clients reconnect. Presence state corrects. Messages sent during partition are delivered. | HIGH |
| F-11 | Malicious client sends forged call:signal_offer to arbitrary user | Server checks: is sender in an active call? Is target in the same call? If not, signal is rejected. Audit log: signal_unauthorized. | No action needed. Authorization layer blocks the attack. Audit trail available for investigation. | CRITICAL |
| F-12 | Client app crash during file upload (50% uploaded) | Server receives partial multipart upload. Upload fails or times out. No file record created in DB. | Client restarts. User re-initiates upload. Server only stores complete uploads (atomic write). | LOW |
| F-13 | mDNS/Zeroconf discovery fails (port 41234 blocked) | Client cannot auto-discover server. Manual IP entry required. | User manually enters server IP:port in the connection settings. Server URL saved in localStorage. | MEDIUM |
| F-14 | Clock skew between server and client (>5 minutes) | JWT validation may fail (iat/exp checks). Message timestamps appear wrong. | Sync system clocks via NTP or local time server. JWT has some tolerance but extreme skew causes auth failures. | MEDIUM |

# 6. Performance Checkpoints

These checkpoints define the quantitative performance requirements for the platform operating on a local network. All measurements assume a clean LAN environment with less than 5ms network RTT between devices.

## 6.1 Latency Benchmarks

| Metric | Target | Measurement Method | Failure Threshold |
| --- | --- | --- | --- |
| Message delivery (end-to-end) | < 500ms | Timestamp diff: sender message:send to receiver message:new | > 2 seconds |
| Message delivery (server processing) | < 50ms | Server log: time between socket event received and emit to recipients | > 200ms |
| Audio call setup time | < 3 seconds | Time from call:initiate to RTCPeerConnection.connectionState = 'connected' | > 10 seconds |
| Video call setup time | < 5 seconds | Same as audio + video track negotiation | > 15 seconds |
| Screen share start time | < 3 seconds | Time from presenter:start to first frame rendered on receiver | > 8 seconds |
| Audio latency (mouth-to-ear) | < 300ms | Clap test: record clap on sender mic, measure delay on receiver speaker output | > 500ms |
| Presence update propagation | < 2 seconds | Time from socket connect/disconnect to presence event received by all clients | > 5 seconds |
| Token refresh round-trip | < 500ms | Time from 401 response to new access token obtained and original request retried | > 2 seconds |
| Socket.IO reconnection | < 5 seconds | Time from WiFi restore to authenticated socket connection | > 30 seconds |
| REST API response (simple) | < 100ms | P95 response time for GET /users/me under normal load | > 500ms |

## 6.2 Throughput Benchmarks

| Metric | Target | Test Method | Failure Threshold |
| --- | --- | --- | --- |
| Messages per second (single channel) | > 50 msg/sec | User sends messages in a loop via socket, count delivered to receiver per second | < 15 msg/sec |
| Concurrent WebRTC streams (group call) | 8 users stable (28 peer connections) | Group call with 8 participants, all audio+video. Monitor CPU, memory, packet loss | Any participant drops below 15 FPS video or experiences > 5% audio packet loss |
| File upload throughput | > 10 MB/sec on LAN | Upload 100MB file, measure time. LAN should not be the bottleneck. | < 2 MB/sec |
| Concurrent socket connections | > 100 total connections | Spin up 100 authenticated python-socketio clients, send presence events | Server crash or > 10 second response time |
| Database write throughput | > 200 inserts/sec | Bulk message inserts via API, measure wall clock time | < 50 inserts/sec |

## 6.3 Resource Utilization Limits

| Resource | Acceptable Range | Monitoring Method | Alert Threshold |
| --- | --- | --- | --- |
| Server CPU usage | < 60% average, < 90% peak | Windows Task Manager or perfmon during test execution | > 90% sustained for > 30 seconds |
| Server RAM usage | < 500 MB baseline + 5 MB/connected user | Process memory monitoring during concurrent test | > 2 GB total or continuous growth (memory leak) |
| Client CPU usage (per app) | < 30% idle, < 70% during call | Electron process monitor (chrome://process-internals or Task Manager) | > 80% sustained outside of a call |
| Client RAM usage (per app) | < 200 MB idle, < 500 MB during video call | Electron process memory | > 800 MB or continuous growth |
| SQLite database file size | < 100 MB per 100K messages | File size on disk after bulk message test | > 500 MB per 100K messages |
| Network bandwidth (audio call) | < 100 kbps per participant | Wireshark or netstat during audio call | > 200 kbps per participant |
| Network bandwidth (video call) | < 2 Mbps per participant | Wireshark during video call | > 5 Mbps per participant |

# 7. Pass/Fail Criteria

## 7.1 Per-Test-Case Criteria

Each test case (TC-01 through TC-12) is graded independently:

| Verdict | Criteria |
| --- | --- |
| PASS | ALL steps in the test procedure produce the expected result. No deviations, no errors in server logs related to the test. |
| CONDITIONAL PASS | All critical steps pass, but 1-2 non-critical steps have minor deviations that do not affect core functionality (e.g., UI cosmetic issue, timing slightly above target but below failure threshold). |
| FAIL | One or more steps produce an unexpected result: data loss, crash, security bypass, incorrect state, or performance above the failure threshold. |
| BLOCKED | Test cannot execute due to environment issue, dependency failure, or previous test failure leaving the system in an unrecoverable state. |

## 7.2 Release Gate Criteria

The following criteria must ALL be met before the build is approved for production deployment:

| Gate | Requirement | Evidence |
| --- | --- | --- |
| Functional Completeness | TC-01 through TC-12 all PASS or CONDITIONAL PASS. Zero FAIL verdicts on P0 tests. | Signed test execution report |
| Security Baseline | All CRITICAL and HIGH findings from SECURITY-ARCHITECTURE.md are verified fixed. No new CRITICAL findings. | Security scan results + manual verification |
| Performance Acceptance | All latency benchmarks (Section 6.1) met. All throughput benchmarks (Section 6.2) met. Resource utilization within limits (Section 6.3). | Performance measurement logs |
| Data Integrity | Zero message loss across all messaging tests. Zero data corruption in persistence tests (TC-10). Unicode content preserved. | Database query results + message comparison |
| Resilience Baseline | WiFi interruption recovery (TC-08) PASS. Server restart recovery (TC-09) PASS. Presence accuracy after disruption verified. | Test execution logs |
| Failure Handling | All CRITICAL and HIGH severity failure scenarios (F-01, F-03, F-06, F-08, F-10, F-11) handled gracefully with documented recovery. | Failure scenario test results |
| Zero Regression | No previously passing test now fails. Automated regression suite (when implemented) passes at 100%. | CI pipeline results |

## 7.3 Defect Classification

| Severity | Definition | Release Impact |
| --- | --- | --- |
| CRITICAL | Data loss, security bypass, complete feature failure, crash. Example: messages not delivered, unauthorized file access, server crash under normal load. | RELEASE BLOCKED. Must fix before deployment. |
| HIGH | Major feature degradation, significant performance issue, intermittent data inconsistency. Example: call drops after 2 minutes, presence ghost entries. | RELEASE BLOCKED for P0 features. Acceptable for P2 features with mitigation plan. |
| MEDIUM | Minor feature issue, cosmetic defects affecting usability, performance above target but below failure threshold. Example: avatar not loading, 600ms message delivery. | Release proceeds with defect tracked. Fix in next patch. |
| LOW | Cosmetic only, edge case with workaround, documentation gaps. Example: tooltip typo, border rendering difference across DPI settings. | Release proceeds. Fix opportunistically. |

# 8. Test Execution Tracker

Use the table below to record results during test execution. Fill in the Tester, Date, Verdict, and Notes columns for each test case.

| Test ID | Scenario | Tester | Date | Verdict | Notes / Defect IDs |
| --- | --- | --- | --- | --- | --- |
| TC-01 | Private Text Chat |  |  |  |  |
| TC-02 | Private Audio Call |  |  |  |  |
| TC-03 | Private Video Call |  |  |  |  |
| TC-04 | Private Screen Sharing |  |  |  |  |
| TC-05 | Group Messaging (3-20) |  |  |  |  |
| TC-06 | Group Audio/Video Call (3-8) |  |  |  |  |
| TC-07 | Group Screen Sharing |  |  |  |  |
| TC-08 | WiFi Interruption Recovery |  |  |  |  |
| TC-09 | Server Restart Recovery |  |  |  |  |
| TC-10 | Message Persistence |  |  |  |  |
| TC-11 | Presence Updates |  |  |  |  |
| TC-12 | Multiple Simultaneous Rooms |  |  |  |  |